afname = electricity taken from the grid / demand / consumption 0-3

opwek = electricity produced and fed into the grid / generation 0-3

When is  “congested”

Conjestged rule: some times  local postcode shows avg_afname_code < 1,

but the larger feeding area still has a problem, such as queue_afname_mw > 0 or utilization_afname_percent >= 90%

Planned grid project:

- EHVN_2027 (2027)

- EHVW_2026 (2026); EHVW_2028 (2028).

With the current data, we cannot see exact remaining capacity for each PC6  5611AS.

We only have remaining capacity at the bigger feeding-area level

Macthing data with buurt Strijd-S
Using buurten, we get exact sape of Strijd-S.

Match close postcode to each neighbourhood

Ex: Strijd-S 5617, takes all  PC6 rows in congestie_pc6.csv that start with that postcode: 5617

And sumizse data

- max_afname_code

- avg_afname_code share_af

- name_code_gt0

- main feeding area

The shape of Strijp-S is exact. The congestion value for Strijp-S is estimated from postcode 5617.

- postcode-based neighbourhood approximation

So the data we try to combine just approximation.

Calculate Nominal Grid Capacity

remaining capacity = available afname capacity - needed afname capacity

use congestie_pc6 to get all PC6 in Strijd S, electricity grid area that supplies power are EHVN (25), EHVW (1). So, only look at EHVN

For EHVN, the source data says:

aanwezige_transportcapaciteit_afname = 111,14 MW

needed_transportcapaciteit_afname = 102,3191745576873 MW

= 8.82MW

111.14 – 102 = 8.82